State of Palestine

Palestinian Central Bureau of Statistics

Emissions to Air, 2014

June, 2016

PAGE NUMBERS OF ENGLISH TEXT ARE PRINTED IN SQUARE BRACKETS.

TABLES ARE PRINTED IN THE ARABIC ORDER (FROM RIGHT TO LEFT)

This document is prepared in accordance with the standard procedures stated in the Code of Practice for Palestine Official Statistics 2006

© June, 2016.

All rights reserved.

Citation:

Palestinian Central Bureau of Statistics, 2016. Emissions to Air, 2014.

Ramallah - Palestine.

All correspondence should be directed to:

Palestinian Central Bureau of Statistics

P.O.Box 1647 Ramallah, Palestine.

Tel: (972/970) 2 298 2700

Fax: (972/970) 2 298 2710

Toll Free: 1800300300

E-Mail: diwan@pcbs.gov.ps

| web-site: http://www.pcbs.gov.ps | Reference ID: 2205 |
| --- | --- |

Acknowledgment

The Palestinian Central Bureau of Statistics (PCBS) extends its deep appreciations to all

workers in the preparation of this report.

The Emissions to Air, 2014 report was planned and prepared by a technical team from PCBS and with joint funding by Palestine and the Core Funding Group (CFG) for the year 2016 represented by the Representative Office of Norway to Palestine and the Swiss Development and Cooperation Agency (SDC).

Moreover, PCBS very much appreciates the distinctive efforts of the Core Funding Group (CFG) for their valuable contribution to funding the report.

Team Work

- Report Preparation

Zahran Khaleef

- Dissemination Standard

Hanan Janajreh

- Preliminary Review

Mahmoud Abd-Alrhman

- Final Review

Mohammad Qalalweh

- Overall Supervision

Ola Awad                         	          President of PCBS

Table of Contents

| # Subject |  | # Page |
| --- | --- | --- |
|  | # List of Tables |  |
|  | # Introduction |  |
| # Chapter One: | # Main Findings | # [13] |
|  | 1.1 Greenhouse Gas (GHG) Inventory for 2014 | # [13] |
|  | 1.1.1 Emissions of CO2, CH4 and N2O | # [13] |
|  | 1.1.2 Emissions from Illegal Israeli Settlements | # [16] |
|  | 1.1.3 Indirect GHGs and F-Gases | # [16] |
|  | 1.1.4 Overall GHG emissions time-series, 2006-2014 | # [16] |
|  | 1.2 Greenhouse Gas Inventory by Sector | # [17] |
|  | 1.2.1 Emissions from Energy | # [17] |
|  | 1.2.2 Emissions from Industrial Process and Product Use (IPPU) | # [19] |
|  | 1.2.3 Emissions from Agriculture, Forestry and Other Land Use (AFOLU) | # [19] |
|  | 1.2.4 Emissions from Waste | # [22] |
|  | 1.3 Key Category Analysis | # [24] |
| # Chapter Two: | # Methodology and Data Quality | # [25] |
|  | - GHG inventory limitations | # [25] |
|  | 2.2 Data Collection | # [25] |
|  | 2.3 Quality Control | # [26] |
| # Chapter Three: | # Concepts and Definitions | # [29] |
|  | # References | # [33] |

# List of Tables

| Table |  | ## Page |
| --- | --- | --- |
| Table 1: | Per capita emissions of CO2 in Palestine and neighboring countries | # [13] |
| Table 2: | National greenhouse gas inventory, 2014 | # [15] |
| Table 3: | Overall GHG emissions by Year (1000 ton CO2 eq.) | # [16] |
| Table 4: | GHG emissions from Energy Sector by Year (1000 ton CO2 eq.) | # [19] |
| Table 5: | GHG emissions from AFOLU Sector by Year (1000 ton CO2 eq.) | # [21] |
| Table 6: | GHG emissions from Waste Sector by Year (1000 ton CO2 eq.) | # [23] |
| Table 7: | Key Categories Analysis for Palestine’s GHG Inventory, 2014 | # [24] |
| Table 8: | GWPs used (IPCC SAR) | # [25] |
| Table 9: | Comparison of Reference and Sectoral Approach | # [27] |

Introduction

Green House Gas Emission inventories are important environmental statistics. An emission inventory gives an overview of emissions of various emittants per source and/or sector. It may cover a certain region or the whole country.

The State of Palestine encounters many threats related to the climatic change; these threats affect water supply, agricultural productivity, biodiversity, human health and the economy. The potential consequences of climatic change radically in light of the gradual desertification and increased resources degradation and scarcity.

Climatic change is a global problem, and addressing it is a global responsibility. Developed countries, which are responsible for the greenhouse gases accumulating in the atmosphere, should take the lead in reducing greenhouse gas emissions.  Developing countries, including Arab countries should adopt low-emission development strategies, including strategies for the reduction in energy and water consumption and the production of waste.

The main objective of this report is to provide estimations and calculations of emissions in air in Palestine.  The methodology depends on the source of pollution and is classified into:

- Emissions from energy

- Emissions from industrial processes and product use (IPPU)

- Emissions from agriculture, forestry, and other land use (AFOLU)

- Emissions from waste

This report is divided into three chapters: the first chapter defines the main findings of the report.  The second chapter explains the methodology of data collection and tabulation, in addition to details regarding data quality and estimates of data sources.  The third chapter covers the concepts and their definition.

| June, 2016 |  | Ola Awad President of PCBS |
| --- | --- | --- |

# Chapter One

# Main Findings

This chapter presents the main findings of emission calculations in Palestine for the year 2014 beside the time series data for the years 2006-2014.

1.1 Greenhouse Gas (GHG) Inventory for 2014

# In 2014, Palestine emitted 4,614,850 metric tons CO2 eq., that's mean Palestine’s emissions per capita are approximately 1.014 tons CO2 eq., where as this figure was 0.8 ton/ capita in 2011. The state of Palestine’s emissions per capita are significantly lower than its neighboring countries. The list below provides the approximate per capita emissions of these countries in CO2 equivalents for comparison:

Table 1: Per capita emissions of CO2 in Palestine and neighboring countries

| Country | Population | Emissions per capita CO2 eq. | Year |
| --- | --- | --- | --- |
| Palestine | 4,550,368 | 1.014 | 2014 |
| Egypt | 66,137,000 | 2.9 | 2000 |
| Syria | 21,533,000 | 3.7 | 2005 |
| Lebanon | 4,160,000 | 4.4 | 2000 |
| Jordan | 5,600,000 | 5.1 | 2006 |
| Israel | .. | 10.7 | 2007 |

Source: Countries communication reports

..: data not available

1.1.1 Emissions of CO2, CH4 and N2O

# The amount of carbon dioxide emitted from the energy, agriculture and solid waste sectors during 2014 was estimated at around 3.18 million tons, whereas the amount of methane emitted was estimated at around 852.4 thousand ton of CO2 equivalent, and the amount of N2O emitted was 582.1 thousand ton of CO2 equivalent.

Greenhouse Gas Inventory for 2014 by Emittant (Thousand ton of CO2 eq.)

# Concerning the distribution of the Greenhouse Gas Inventory for 2014 by sector, it was distributed into 3,309.58 thousand tons of CO2 eq. emitted from the energy sector, 503.65 thousand ton of CO2 eq. from agriculture and the remaining 801.62 thousand ton of CO2 eq. from the solid waste sector (open burning of solid waste).

Greenhouse Gas Inventory for 2014 by sector (Thousand ton of CO2 eq.)

Table 2: National greenhouse gas inventory, 2014

| Categories | CO2 (Gg) | CH4 (Gg) | N2O (Gg) | CO (Gg) | NOx (Gg) | NMVOCs (Gg) | SOX (Gg) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Total National Emissions and Removals | 3,180.3 | 40.6 | 1.9 | NE | NE | NE | NE |
| 1 - Energy | 3,211.9 | 2.4 | 0.2 | NE | NE | NE | NE |
| 1.A - Fuel Combustion Activities | 3,211.9 | 2.4 | 0.2 | NE | NE | NE | NE |
| 1.A.1 - Energy Industries | 244.6 | 0.0 | 0.0 | NE | NE | NE | NE |
| 1.A.2 - Manufacturing Industries and Construction | 52.1 | 0.1 | 0.0 | NE | NE | NE | NE |
| 1.A.3 - Transport | 2,364.9 | 0.4 | 0.1 | NE | NE | NE | NE |
| 1.A.4 - Other Sectors | 550.4 | 1.9 | 0.0 | NE | NE | NE | NE |
| 1.A.5 - Non-Specified | 0.0 | 0.0 | 0.0 | NE | NE | NE | NE |
| 1.B - Fugitive emissions from fuels | 0.0 | 0.0 | 0.0 | NE | NE | NO | NO |
| 1.B.1 - Solid Fuels | 0.0 | 0.0 | 0.0 | NE | NE | NO | NO |
| 1.B.2 - Oil and Natural Gas | 0.0 | 0.0 | 0.0 | NE | NE | NO | NO |
| 2 - Industrial Processes and Product Use | NO | NO | NO | NE | NE | NE | NE |
| 2.A - Mineral Industry | NO | NO | NO | NE | NE | NE | NE |
| 2.B - Chemical Industry | NO | NO | NO | NE | NE | NE | NE |
| 2.C - Metal Industry | NO | NO | NO | NE | NE | NE | NE |
| 2.D - Non-Energy Products from Fuels and Solvent Use | NO | NO | NO | NE | NE | NE | NE |
| 2.G - Other Product Manufacture and Use | NO | NO | NO | NE | NE | NE | NE |
| 3 - Agriculture, Forestry, and Other Land Use | -31.6 | 8.7 | 1.1 | NE | NE | NE | NE |
| 3.A - Livestock | 0.0 | 8.7 | 0.0 | NE | NE | NE | NE |
| 3.A.1 - Enteric Fermentation | NO | 7.6 | NO | NE | NE | NE | NE |
| 3.A.2 - Manure Management | NO | 1.1 | 0.0 | NE | NE | NE | NE |
| 3.B - Land | -31.6 | 0.0 | 0.0 | NE | NE | NE | NE |
| 3.B.1 - Forest land | -31.6 | NE | NE | NE | NE | NE | NE |
| 3.C - Aggregate sources and non-CO2 emissions sources on land | 0.0 | 0.0 | 1.1 | NE | NE | NE | NE |
| 3.C.4 - Direct N2O Emissions from managed soils | NE | NE | 1.1 | NE | NE | NE | NE |
| 3.C.5 - Indirect N2O Emissions from managed soils | NE | NE | 0.0 | NE | NE | NE | NE |
| 3.C.6 - Indirect N2O Emissions from manure management | NE | NE | 0.1 | NE | NE | NE | NE |
| 3.C.7 - Rice cultivations | NE | NO | NE | NE | NE | NE | NE |
| 3.D - Other | 0.0 | 0.0 | 0.0 | NE | NE | NE | NE |
| 3.D.1 - Harvested Wood Products | NE | NE | NE | NE | NE | NE | NE |
| 4 - Waste | 0.0 | 29.5 | 0.6 | NE | NE | NE | NE |
| 4.A - Solid Waste Disposal | 0.0 | 13.4 | 0.0 | NE | NE | NE | NE |
| 4.B - Biological Treatment of Solid Waste | NO | NO | NO | NE | NE | NE | NE |
| 4.C - Incineration and Open Burning of Waste | 0.0 | 0.0 | 0.0 | NE | NE | NE | NE |
| 4.D - Wastewater Treatment and Discharge | 0.0 | 16.0 | 0.6 | NE | NE | NE | NE |

# Gg: Giga gram which equals 1000 ton

# NO: Not Occurring

# NE: Not Estimated

# Table 2 is the standard reporting table of direct and indirect GHG emissions. The table provides a complete list of direct GHGs in each sector for the year 2014.

1.1.2 Emissions from Illegal Israeli Settlements

Data showed that there were 150 Israeli settlements in the West Bank by the end of 2014, lodging over 599,901 settlers.  According to data of Israel’s second national communication, the emissions per capita were 10.7 tons of CO2 eq. in 2007.  From these data, the emissions of Israeli settlers in Palestine were estimated at 6,418,941 metric tons of CO2 eq.  Israeli emissions on Palestinian land are higher by 140% than the total emissions of the state of Palestine.

1.1.3 Indirect GHGs and F-Gases

The emissions of the indirect GHGs (CO, NOx, NMVOCs and SOx) in table 2 have not been estimated.  It is unnecessary for Palestine to estimate and report on such emissions, as they are a non-Annex I country, another reason for not calculating these emissions is the absence of their emission factors.

Moreover, the national communication recommends that each country also include a table on the national greenhouse gas inventory of anthropogenic emissions of HFCs, PFCs, and SF6; see Table 3. In Palestine, emissions of F-gases will arise from a range of sources (such as air conditioning, and refrigeration) but no activity data are available on these to make accurate estimates or to quantify these emissions. However, it is the view of the Palestine GHG inventory team that the emissions from sources using F-gases will be very small both in absolute terms and as a GWP weighted fraction of the national total emissions. This view is based on the fact that typically total F-gas emissions are approximately 1 to 3 % of national net GWP weighted emissions.

1.1.4 Overall GHG emissions time-series, 2006-2014

The table and figure below show the overall emission trends from 2006-2014 in Palestine.

Table 3: Overall GHG emissions by Year (1000 ton CO2 eq.)

| Year | Overall GHG emissions (1000 ton) |  |  | Overall GHG emissions (1000 ton CO2 eq.) |
| --- | --- | --- | --- | --- |
|  | Net CO2 | CH4 | N2O |  |
| 2006 | 1,510.6 | 33.4 | 1.7 | 2,739.9 |
| 2007 | 1,479.6 | 33.6 | 1.7 | 2,710.7 |
| 2008 | 1,374.4 | 33.9 | 1.7 | 2,620.3 |
| 2009 | 1,779.2 | 34.0 | 1.7 | 3,026.6 |
| 2010 | 2,049.7 | 35.7 | 1.5 | 3,276.5 |
| 2011 | 1,900.2 | 38.2 | 1.7 | 3,226.3 |
| 2012 | 2,059.3 | 38.5 | 1.7 | 3,380.6 |
| 2013 | 2,294.7 | 38.5 | 1.6 | 3,612.0 |
| 2014 | 3,180.3 | 40.6 | 1.9 | 4,614.9 |

Overall GHG emissions by Year (1000 ton CO2 eq.)

Emissions steadily increase over time but there is no direct relationship between time and emissions mainly because the majority of emissions come from fuel combustion in the energy sector and the consumption differs from year to year. Many variables control the fuel consumption such as the variability in the seasonal temperatures, the situation in Gaza, the unmeasured quantities of fuel used and other statistical differences between fuel supply and demand.

The energy and waste sectors are the main drivers of that increase as the emissions from the AFOLU sector decrease over time. The use of fuel in the energy sector is responsible for driving the increase of CO2 while activity in the waste sector (including waste disposal and wastewater treatment) is responsible for driving the increase of CH4.

One explanation for the overall rise of emissions in Palestine is the increasing population, which drives increasing energy consumption in general, as well as emissions from increasing transportation activity. Moreover, with a growing population, waste disposal and wastewater generation increase as well. However, wastewater is responsible for the main increase in CH4 emissions. Wastewater treatment is not yet fully regulated and the wastewater treatment systems are in some cases old and poorly maintained. Waste disposal has been to some extent controlled with the introduction of new sanitary landfills, the closing of old dumpsites and the banning of illegal dumping.

1.2 Greenhouse Gas Inventory by Sector

1.2.1 Emissions from Energy

During 2014, the energy sector emitted 3.31 million ton of CO2 eq. and dominates the GHG emissions in Palestine. Within the energy sector, CO2 emissions dominate, with relatively very small emissions of CH4 and N2O. All the emissions arise from the combustion of fuel.

Emissions from the energy sector by Emittant, 2014 (1000 ton CO2 eq.)

Within the energy sector, the subsectors that contribute to GHG emissions are the following:

- Transport: Emissions from transport are almost half of all energy sector emissions (73%). It is important to note that the estimates are only for the road transportation sector. There is no domestic aviation in Palestine.

- Other Sectors: The “Other sectors” subcategory is the second largest contributor to GHG emissions in the energy sector. It refers to energy use in the commercial/institutional (2%) and residential (98%) sectors as well as emissions from stationary and off-road vehicles and other machinery for the Agriculture/Forestry industry. Emissions from the Agriculture/Forestry sector are negligible.

- Energy Industries: The energy industry sector contributes 8% of emissions in the energy sector. This refers specifically to electricity generation only.

- Manufacturing industries and construction: The emissions reflects the aggregated emissions from all relevant industries in Palestine and contribute very little to the overall energy emissions.

- Agriculture /Forestry / Fishing: Emissions from fishing vessels are trivial and have not currently been included in the inventory.

Emissions from the energy sector by subsector, 2014 (1000 ton CO2 eq.)

Time-series for energy sector 2006-2014

The time series for the energy sector shows an overall increasing trend in emissions. The activity data for all the years 2006-2014 comes from Palestine’s Energy Balance provided by PCBS. The main GHG driving this increase in emissions is CO2. A potential explanation for the increasing trend is the growing population and hence the growing use of transportation as well as the increased energy use from residential.

Table 4: GHG emissions from Energy Sector by Year (1000 ton CO2 eq.)

| Year | GHG emissions (1000 ton) |  |  | GHG emissions (1000 ton CO2 eq.) |
| --- | --- | --- | --- | --- |
|  | Net CO2 | CH4 | N2O |  |
| 2006 | 1,541.36 | 1.02 | 0.05 | 1,578.50 |
| 2007 | 1,510.25 | 1.14 | 0.05 | 1,550.38 |
| 2008 | 1,404.87 | 1.08 | 0.05 | 1,443.53 |
| 2009 | 1,809.55 | 1.35 | 0.06 | 1,857.82 |
| 2010 | 2,080.06 | 2.14 | 0.09 | 2,151.70 |
| 2011 | 1,930.85 | 2.04 | 0.08 | 1,997.73 |
| 2012 | 2,090.28 | 2.00 | 0.08 | 2,156.93 |
| 2013 | 2,330.54 | 2.01 | 0.10 | 2,403.54 |
| 2014 | 3,211.92 | 2.41 | 0.15 | 3,309.58 |

1.2.2 Emissions from Industrial Process and Product Use (IPPU)

Palestine has no “heavy industry”, and some, but not extensive, “light industry”. Palestine conducted an analysis of the type of industries that are present in the countries, ranking them by number of employed persons, number of enterprises, sum of fuel and oil use, and sum of electricity use. The aim was to identify the major industries in Palestine, and find out whether they could emit GHG – as process emissions. Two of the industries present in Palestine that could produce emissions are the “manufacture of food products” and “manufacture of basic metals.” However, none of these industries are mentioned in the IPCC 2006 guidelines as having direct GHGs emissions.

Moreover, Palestine does not contain chemical, metals, or electronic industries. The 2014 inventory therefore does not include any emissions from this sector.

1.2.3 Emissions from Agriculture, Forestry and Other Land Use (AFOLU)

Net emissions in the AFOLU sector was estimated at 503.7 thousand tons of CO2 eq. and is the only sector where carbon sinks occur. There is a small net sink of 31.6 thousand tons of CO2 eq.

Emissions of N2O emissions dominate, followed by CH4. Emissions of CH4 arise from manure management and enteric fermentation.  N2O emissions arise from fertilizer application to soils, and, from indirect emissions associated with manure management.

Emissions from AFOLU sector by Emittant, 2014 (1000 ton CO2 eq.)

Within the AFOLU sector, the subsectors that contribute to GHGs emissions and removals are the following:

- Land: Emissions and removals of CO2 can occur in the AFOLU sector. To estimate the emissions and removals the calculations were based on land areas that were stratified by climate regions. Palestine identified all available data on land area, use, climatic zone and vegetation type. The country was divided into three zones: West Bank including East Jerusalem and, Gaza. Then, Palestine was segregated into five specific climate regions. The climatic regions were:

- Central Highlands region

- Coastal plain (Gaza)

- Eastern slopes region

- Jordan valley region

- Semi coastal region

These categories were then mapped to the nearest corresponding IPCC categories: Forestland, Cropland, Grassland, Wetlands, Settlements and Other Land. See “Annex B: Supplementary information” for more details. The current inventory does not account for land use changes, as land use change data were not available.

- Livestock: Livestock in Palestine consists of dairy cows, sheep, goats, camels and poultry. Sheep and poultry however are the two most dominant types of animals in Palestine. These along with the other livestock are responsible for producing a significant amount of methane. This is mainly due to enteric fermentation and manure management; there are emissions of N2O from manure management also.

There are two kinds of manure management practices in Palestine.

- The first where manure is allowed to lie as deposited and is not managed known as “pasture/range/paddock”.

- The second where manure is routinely removed and is applied to cropland or pasture within 24 hours known as “daily spread”.

The type of manure management system greatly affects the amount of methane and nitrous oxide emitted.

- Aggregate sources and non-CO2 emissions sources on land: This subsector only produces N2O due to the use of ammonium sulfate as a fertilizer.

- Other: The “other” subsector refers to “harvested wood products” and represents a carbon sink. The sink of carbon associated with harvested wood products is automatically calculated from the quantity of wood, which has been disposed into SWDS from domestic consumption.

.

Emissions from AFOLU sector by Subsector, 2014 (1000 ton CO2 eq.)

Time-series of emissions for AFOLU sector 2006-2014

The AFOLU contains both emissions and removals. The emissions of N2O and CH4 are greater than the carbon sinks which results in overall positive net emissions. However, net emissions in the AFOLU sector decline over time. This is because N2O and CH4 emissions overall decrease over time coupled with carbon sinks being more or less the same over the period.

The decrease in N2O since 2006 could potentially be explained by using less fertilizer on the land as a result of Israeli restrictions on the importation of some fertilizers, and the decrease in CH4 since 2006 could be explained by improved manure management practices. The decrease  in carbon sinks since 2006 is explained by assuming land use change decrease over the years.

Table 5: GHG emissions from AFOLU Sector by Year (1000 ton CO2 eq.)

| Year | GHG emissions (1000 ton) |  |  | GHG emissions (1000 ton CO2 eq.) |
| --- | --- | --- | --- | --- |
|  | Net CO2 | CH4 | N2O |  |
| 2006 | -33.83 | 9.43 | 1.18 | 528.75 |
| 2007 | -34.07 | 8.73 | 1.15 | 506.85 |
| 2008 | -34.28 | 8.26 | 1.17 | 501.15 |
| 2009 | -34.45 | 7.28 | 1.13 | 469.46 |
| 2010 | -34.71 | 7.21 | 0.91 | 400.34 |
| 2011 | -35.40 | 8.78 | 1.06 | 476.90 |
| 2012 | -35.66 | 8.05 | 1.00 | 444.78 |
| 2013 | -35.88 | 7.40 | 0.97 | 419.80 |
| 2014 | -31.59 | 8.73 | 1.14 | 503.65 |

GHG emissions from AFOLU Sector by Year (1000 ton CO2 eq.)

1.2.4 Emissions from Waste

In 2014, emissions in the waste sector were estimated 801.616 thousand tons of CO2 eq. Emissions of CH4 dominate, followed by N2O. Emissions of CH4 arise from waste decomposition and wastewater treatment whereas N2O emissions arise during biological nitrogen removal in waste water treatment plants.  A very small amount of CO2 is also emitted which arises from the burning of waste.

Emissions from Waste sector by Emittant, 2014 (1000 ton CO2 eq.)

Within the waste sector, the categories that contribute to GHG emissions are the following:

- Solid Waste Disposal, which produce methane.

- Industrial waste.

- Biological treatment of solid waste: This does not occur in Palestine.

- Incineration and open burning of waste: Incineration of either medical waste or solid municipal waste does not occur in Palestine. However, open burning of waste does and contributes to CO2, CH4, and N2O emissions.  The open burning of waste, although still occurring, has decreased substantially since 2012.  With illegal dumping sites closing down and waste being shifted to legal landfills, waste burning has become a less common method of waste disposal.

- Domestic wastewater treatment and discharge: Palestine uses different wastewater treatment systems. Wastewater treatment and discharge arises from both domestic and industrial sites and produces methane and nitrous oxide. Emissions of N2O are released during biological nitrogen removal in wastewater treatment plants.

Industrial wastewater treatment and discharge: Emissions from industrial wastewater discharge from olive oil mills are also included.

Emissions from Waste sector by Subsector, 2014 (1000 ton CO2 eq.)

Time-series of emissions from the waste sector 2006-2014

The waste sector produces a significant amount of methane emissions, some nitrous oxide and very little CO2.  The trend in emissions from this sector is increasing and this is because with a growing population, waste disposal and wastewater discharge are also increasing.

Table 6: GHG emissions from Waste Sector by Year (1000 ton CO2 eq.)

| Year | GHG emissions (1000 ton) |  |  | GHG emissions (1000 ton CO2 eq.) |
| --- | --- | --- | --- | --- |
|  | Net CO2 | CH4 | N2O |  |
| 2006 | 3.11 | 22.94 | 0.48 | 632.70 |
| 2007 | 3.43 | 23.69 | 0.49 | 653.43 |
| 2008 | 3.79 | 24.51 | 0.51 | 675.58 |
| 2009 | 4.11 | 25.41 | 0.52 | 699.34 |
| 2010 | 4.38 | 26.37 | 0.54 | 724.50 |
| 2011 | 4.74 | 27.40 | 0.55 | 751.71 |
| 2012 | 4.69 | 28.46 | 0.57 | 778.90 |
| 2013 | 0.00 | 29.09 | 0.57 | 788.68 |
| 2014 | 0.00 | 29.45 | 0.59 | 801.62 |

GHG emissions from Waste Sector by Year (1000 ton CO2 eq.)

1.3 Key Category Analysis

Key categories are defined as the sources of emissions that have a significant influence on the inventory as a whole, in terms of the absolute level of the emissions, the trend, or both.

These key source categories have been identified so that the resources available for inventory preparation may be prioritized, and so that the best possible estimates are prepared for the most significant categories that will help in defining areas of mitigations for proper mitigation plans.

Both the level and the trend assessments have been completed, following the procedure set out in the IPCC Good Practice Guidance (2000). This means that the categories that contribute to 95% of the emissions have been identified. The emission estimates were taken from the current inventory.

The table below lists the key categories, for 2014 and the base year, derived from the IPCC Approach key category analysis.

Table 7: Key Categories Analysis for Palestine’s GHG Inventory, 2014

| IPCC Category code | IPCC Category | Greenhouse gas | Effect* | Percentage |
| --- | --- | --- | --- | --- |
| 1.A.3.b | Road Transportation | CO2 | L,T | 51% |
| 1.A.4 | Other Sectors - Liquid Fuels | CO2 | L,T | 11% |
| 4.D | Wastewater Treatment and Discharge | CH4 | L,T | 7% |
| 3.C.4 | Direct N2O Emissions from managed soils | N2O | L,T | 7% |
| 4.A | Solid Waste Disposal | CH4 | L,T | %6 |
| 1.A.1 | Energy Industries - Liquid Fuels | CO2 | L,T | %5 |
| 4.D | Wastewater Treatment and Discharge | N2O | L,T | 4% |
| 3.A.1 | Enteric Fermentation | CH4 | L,T | %3 |
| 1.A.4 | Other Sectors - Solid Fuels | CO2 | L,T | 1% |

*: L: Level Assessment, T: Trend Assessment

Chapter Two

Methodology and Data Quality

This chapter presents the scientific methodology and quality procedures used in the estimates of emissions to air from their various sources, including the data quality and procedures of data quality.

The estimates of emissions to air is based on the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (2006 Guidelines). These guidelines were produced at the invitation of the United Nations Framework Convention on Climate Change (UNFCCC) to update the Revised 1996 Guidelines and associated good practice guidance, which provide internationally agreed methodologies intended for use by countries to estimate greenhouse gas inventories to report to the UNFCCC.2006.

2.1 GHG inventory limitations

In this inventory, Palestine estimated emissions of GHGs and removals of CO2.  The GHGs included in the inventory were:

- Carbon dioxide (CO2)

- Methane (CH4)

- Nitrous oxide (N2O)

From the following Sectors:

- Energy

- Industrial Processes and Product Use (IPPU)

- Agriculture, Forestry and Other Land Use (AFOLU)

- Waste.

To calculate the Global Warming Potential (GWP) weighted emissions of each gas, each gas is given equivalent weight compared to CO2, Example: the GWP of 1 ton of CH4 is equal to the GWP of 21 ton of CO2,  and so on.  To calculate the GWP, the IPCC Second Assessment Report (SAR) GWP weightings were used.

Table 8: GWPs used (IPCC SAR)

| Gas | CO2 Equivallent (SAR( |
| --- | --- |
| CO2 | 1.0 |
| CH4 | 21.0 |
| N2O | 310.0 |

2.2 Data Collection

To complete Palestine’s first complete GHG inventory, data was collected from various key institutions: the Palestinian Central Bureau of Statistics (PCBS), the Ministry of Local Governments, the Ministry of Agriculture, United Nations Development Program/Program of Assistance to the Palestinian People (UNDP/PAPP), Birzeit University, and the Environment Quality Authority (EQA) in addition to field surveys and administrative records. Most of the data collected has been Activity Data (AD); but in some cases Emission Factors (EFs) were collected. Most effort was placed on identifying the AD rather than EFs primarily because there are adequate default EFs available, but the AD is country specific and needs to be sourced from within Palestine.

2.3 Quality Control

Throughout the data collection and data management processes, Quality Control (QC) measures were applied to identify and remove poor quality data or to identify where more information about provided data was needed. These checks included, inter alia, checks on time series consistency; inter comparisons of one data source with another, assessment of the magnitude of data, checks of “reasonableness”, and checks of implied emission factors against typical values.

Data quality

The United Nations Framework Convention on Climate Change (UNFCCC) asked the Intergovernmental Panel on Climate Change (IPCC) to develop guidelines setting out the approaches countries should take towards implementing “good practice” when developing a greenhouse gas inventory. The goal of the guidelines is to assist countries in producing inventories that are “accurate in the sense of being neither over nor underestimates so far as can be judged, and in which uncertainties are reduced as far as practicable”.

It also upholds the values that an inventory must be transparent, consistent, comparable, complete and accurate.  Palestine has applied these guiding principles in the creation of their GHG inventory. As this is Palestine’s first national communication, it has concentrated on developing a complete inventory that is as accurate as possible.

Palestine has taken the following steps to comply with each of these overarching principles:

- Transparency: The GHG inventory for Palestine has been created in a transparent and traceable way. “Master” spreadsheets in Excel were created for each sector to store and process all the data and make the necessary calculations to “gap fill” where data was missing. To ensure transparency, the master spreadsheets document all the assumptions made, any emission factors used, and all data sources used.

- Accuracy: In an attempt to be as accurate as possible, country specific data was used when possible instead of making simplified assumptions or using default data in the IPCC software.  For instance, in the waste sector, Palestine made an effort to gather the activity data on waste arising between 2006 and 2013 as opposed to simply multiplying population by IPCC default assumptions about waste generation/capita. Other examples of country specific activity data are energy use, waste composition, and animal numbers. However, in some instances where data was available for one year and not others, a range of approaches have been used to gap fill and extrapolate. For example, data on land use was incomplete and hence it was assumed that the land use in 2011 applied to all other years in the inventory.

- Completeness: Given that this is Palestine’s first inventory, Palestine has worked hard to create a complete inventory for all the sectors relevant to the country.

- Comparability: By using the IPCC methods, Palestine’s GHG inventory is comparable with other inventories generated using these methods. This is important because it allows Palestine to compare its inventory to other similar countries.

- Consistency: As the State of Palestine has developed a time series for its inventory, it has ensured that the same method, methodology and assumptions were used where possible across the time series. This helps ensure time series consistency.

Uncertainties

To minimize uncertainty for this inventory, various QC steps have been taken as discussed in the Quality Control section above. The accuracy of the energy statistics are important to the accuracy of the GHG inventory. The RA/SA inter-comparison will be used to help reduce the uncertainty associated with the energy statistics.

Reference/Sectoral Approach

Another important part of conducting QC for the GHG inventory is the calculation of a reference approach (RA) inventory. This was done using the IPCC software. The RA inventory is calculated to verify the emissions from the energy sector. According to the IPCC the “RA is a top-down approach, using a country’s energy supply data to calculate the emissions of CO2 from combustion of mainly fossil fuels”. The RA is then compared to the results of the energy sectoral approach.

The comparison of the RA and sectoral approach (SA) for the energy sector in Palestine showed a 13.8% difference. This difference is mainly due to energy statistics such as statistical differences and uncertainty in fuel consumption and losses.

Table 9: Comparison of Reference and Sectoral Approach

| Variable | RA | SA | Difference, % |
| --- | --- | --- | --- |
| CO2 Emissions, (1000 ton) | 2,768.6 | 3,211.9 | 13.8 |
| Energy Consumed, (Tera Joul TJ) | 39,258.1 | 44,743.9 | 12.3 |

Chapter Three

# Concepts and Definitions

Solid Waste Burning:

Outdoor burning of wastes such as lumber, used textiles, and others.

Agriculture Waste:

Waste produced as a result of various agricultural operations. It includes manure and other waste from farms, poultry houses and slaughterhouses; harvest waste; fertilizer run-off from fields; pesticides that enter into water, air or soil; and salt and silt drained from fields.

Household Waste:

Waste material usually generated in the residential environment.  Waste with similar characteristics may be generated in other economic activities and can thus be treated and disposed together with household waste.

Nitrogen Oxides (NOx):

A group of highly reactive gases that contain nitrogen and oxygen in varying amounts. Many of the nitrogen oxides are colorless and odorless. The common Emittant nitrogen dioxide (NO2) can often be seen combined with particles in the air as a reddish-brown layer over many urban areas. Nitrogen oxides are formed when the oxygen and nitrogen in the air react with each other during combustion. The formation of nitrogen oxides is favored by high temperatures and excess oxygen (more than is needed to burn the fuel). The primary sources of nitrogen oxides are motor vehicles, electric utilities, and other industrial, commercial, and residential sources that burn fuels.

Nitrous oxide (N2O):

A powerful greenhouse gas emitted through soil cultivation practices, especially the use of commercial   and organic fertilizers, fossil-fuel combustion, nitric acid production, and biomass burning. One of the  six greenhouse gases to be curbed under the Kyoto Protocol.

Emission:

Discharge of Emittants into the atmosphere from stationary sources such as smoke stacks, other vents surface areas of commercial or industrial facilities, and mobile sources, for example, motor vehicles, locomotives and aircraft.

Carbon dioxide (CO2):

A naturally occurring gas, and also a by-product of burning fossil fuels and biomass, as well as land-use changes and other industrial processes. It is the principal anthropogenic greenhouse gas that affects  the Earth’s radiative balance. It is the reference gas against which other greenhouse gases are measured and therefore has a Global Warming Potential of 1.

Equivalent CO2 (carbon dioxide):

The concentration of carbon dioxide that would cause the same amount of radiative forcing as a given mixture of carbon dioxide and other greenhouse gases.

Greenhouse gas:

Greenhouse gases are those gaseous constituents of the atmosphere, both natural and anthropogenic,  that absorb and emit radiation at specific wavelengths within the spectrum of infrared radiation emitted by the Earth’s surface, the atmosphere, and clouds. This property causes the greenhouse effect.  Water vapor (H2O), carbon dioxide (CO2), nitrous oxide (N2O), methane (CH4), and ozone (O3) are the  primary greenhouse gases in the Earth’s atmosphere.  Moreover there are a number of entirely human- made greenhouse gases in the atmosphere, such as the halocarbons and other chlorine- and bromine- containing substances, dealt with under the Montreal Protocol. Besides CO2, N2O, and CH4, the Kyoto Protocol deals with the greenhouse gases sulfur hexafluoride (SF6), hydro fluorocarbons (HFCs), and  per fluorocarbons (PFCs).

Methane (CH4):

A hydrocarbon that is a greenhouse gas produced through anaerobic (without oxygen)  decomposition of waste in landfills, animal digestion, decomposition of animal  wastes, production and distribution of natural gas and oil, coal production, and  incomplete fossil-fuel combustion. Methane is one of the six greenhouse gases to be  mitigated under the Kyoto Protocol.

Ozone (O3):

Pungent, colorless, toxic gas that contain three atoms of oxygen is each molecule.  It occurred naturally at a concentration of about 0.01 parts per million (p.p.m) of air.  Levels of 0.1 p.p.m. are considered to be toxic.

Carbon Monoxide (CO):

Colourless, odourless and poisonous gas produced by incomplete fossil fuel combustion.  Carbon monoxide combines with the hemoglobin of human beings, reducing its oxygen carrying capacity, with effects harmful to human beings.

Sulphur Dioxide (SO2):

Heavy, pungent, colorless gas formed primarily by the combustion of fossil fuels.  It is harmful to human beings and vegetation, and contributes to the acidity in precipitation.

Suspended Particular Matter (SPM):

Finely divided solids or liquids that may be dispersed through the air from combustion processes, industrial activities or natural sources.

Particulate:

Fine liquid or solid particles, such as dust, smoke, mist, fumes, or smog, found in air or emissions.

Organic compounds:

Compounds containing carbon (including carbonates, bicarbonates, carbon dioxide and carbon monoxide) that form the basis of living mattes.  In domestic sewage, organics one mainly metabolic wastes of faeces or urine plus grease, detergents and so forth.

Volatile organic compounds (VOCs):

Organic compounds that evaporate readily and contribute to air pollution mainly through the production of photochemical oxidants.

Air Pollution Sources:

Activities that result in air pollution including agricultural activities, combustion processes, dust producing processes, manufacturing activities, nuclear energy-related activities, spray-painting, printing, dry-cleaning and so on.

Air Quality Standards:

Levels of air Emittants prescribed by regulations that may not be exceeded during a specified time in a defined area.

Emission standard:

Maximum amount of polluting discharge legally allowed from a single source, mobile or stationary.

Air Emittants:

Substances in air that could, at high enough concentrations, harm human beings, animals, vegetation or material.  Air Emittants may thus of being airborne.  They may consist of solid particles, liquid droplets or gases, or combinations of these forms.

Hydrocarbons:

Compounds of hydrogen and carbon in various combinations that are present in petroleum products and natural gas.  Some hydrocarbons are major air Emittants, some may be carcinogenic and others contribute to photochemical smog.

Charcoal:

Solid residue consisting mainly of carbon obtained by the destructive distillation of wood in the absence of air.

Fossil fuel:

Cool, oil and natural gas.  They are derived from the remains of ancient plant and animal life.

Municipal Solid Waste (MSW):

Municipal waste is generally defined as waste collected by municipalities or other local authorities. However, this definition varies by country. Typically, MSW includes: household waste; garden (yard) and park waste; and commercial/institutional waste.

Hazardous waste:

Waste oil, waste solvents, ash, cinder and other wastes with hazardous nature, such as flammability, explosiveness, causticity, and toxicity, are included in hazardous waste. Hazardous wastes are generally collected, treated and disposed separately from non-hazardous MSW and industrial waste streams.  Some hazardous wastes are incinerated and can contribute to the fossil CO2 emissions from incineration.

References

- IPCC 2006, IPCC Guidelines for National Greenhouse Gas Inventories, Prepared by the National Greenhouse Gas Inventories Programme, Eggleston H.S., Buendia L., Miwa K., Ngara T. and Tanabe K. (eds).  Published: IGES, Japan.

- Palestinian Central Bureau of Statistics.  Energy Balance in the Palestinian Territory 2010.  Ramallah- Palestine.

- The Applied Research Institute – Jerusalem (ARIJ), 2011.  State of the Environment in the Palestinian Territory – A Human Rights - Based Approach 2011.  Bethlehem – Palestine.

- International Energy Agency website: http://www.iea.org/co2highlights.

- European Commission, Joint Research Centre (JRC)/PBL Netherlands Environmental Assessment Agency. Emission Database for Global Atmospheric Research (EDGAR), release version 4.2. http://edgar.jrc.ec.europe.eu, 2011.

- UNSD Millennium Development Goals Indicators database (see http://mdgs.un.org/unsd/mdg/Data.aspx). United Nations, Department of Economic and Social Affairs, Population Division, World Population Prospects: The 2008 Revision, New York, 2009 (advanced Excel tables). UNSD Demographic Yearbook.

22